Assignment: Watch

Learning goals	2

Introduction	2

Watch design overview	3

Assignments	5

Part A: implementing bit manipulation	5

Part B (optional): implementing the Watch on an Arduino	5

Appendix: Watch device registers	6

Configuration registers	6

0x20: Configuration register (CONF)	6

Time registers	6

0x21: Time value register MSB (Read/Write)	6

0x22: Time value register LSB (Read/Write)	6

Date registers	7

0x23: Date value register MSB (Read/Write)	7

0x24: Date value register LSB (Read/Write)	7

# Learning goals

- Bit manipulation (part A)

- I2C device programming (part B)

- General programming exercise (part C)

# Introduction

This assignment will program an I2C Watch device that stores the current time and date. The I2C device has the user registers as given in table 1.

| Address | Name | Definition |
| --- | --- | --- |
| 0x20 | CONFIG | Configuration |
| 0x21 | TIME_BITS_HIGH | Time value (MSB) |
| 0x22 | TIME_BITS_LOW | Time value (LSB) |
| 0x23 | DATE_BITS_HIGH | Date value (MSB) |
| 0x24 | DATE_BITS_LOW | Date value (LSB) |

Table 1: User registers of I2C watch device

The device has one register for configuring the watch (CONF) and four other registers for storing the date and time. Both date and time are stored in a 16 bits using two byte registers.

The meaning of the registers in table 1 can be found in the appendix.

# Watch design overview

In figure 1 you can find a possible design for the watch. This design assumes that code is running on a device like an PC or Arduino an communicates via I2C with the Watch device.

In order to get the watch running you will have to implement the ‘watch_register’ module. The other modules are already available.

Figure 1: Code modules of the watch design

In table 2 you can find an overview of the modules.

| Module | Summary |
| --- | --- |
| main | Contains the main method. Show example code on how to use the watch |
| watch | Control the watch |
| watch_registers | Manipulates the register bytes of the watch according the register specification (see appendix) |
| watch_i2c | Sending and retrieving bytes to the Watch device via I2C. |

Table 2: Modules of the watch design

In order to decrease development time a simulator for the Watch device is used. The module design using the simulator is show in figure 3.

Figure 3: Code modules of the watch design using a simulator

| Module | Summary |
| --- | --- |
| watch_device_simulator | Contains a simulator for the I2C watch device. The simulator makes use of the module ‘watch_registers’ for manipulating the register values. |

Table 3: Modules of the watch design using a watch simulator

For making this assignment you can make use of the ‘Watch’ project code. This project can be compiled and executed on a PC. You can find the build instructions in table 4.

| Build instructions: | Description: |
| --- | --- |
| make | Builds the executable called ‘build/main. You can run it by executing ./build/main in your terminal. |
| make test | Builds (build/main_test) and executes the unit tests. |

Table 4: Build instructions

# Assignments

## Part A: implementing register manipulation

Implement the methods as found in ‘shared/watch_registers.h’ for manipulating the watch device registers. Also complete the unit tests in ‘test/watch_registers_test.c’.

Tips:

- Also make sure to read the method documentation in ‘watch_registers.h’ before you start. When for instance setting the hours in the in the TIME_BITS_LOW and TIME_BIT_HIGH you should leave the minute en second bits unchanged. Also note the precondition (@pre) that states that this method may NOT be called with NULL parameters.

- Make sure to use the register specification as can be found in the Appendix.

Note: for implementing the ‘set’ methods it is not allowed to use the ‘get’ methods. This would decrease the learning effect of this exercise!

## Part B (optional): migrating the Watch to Arduino platform (Master + Slave)

Note: this part is likely to more time consuming than part A.

Implement the Watch device on an Arduino. In order to do so you will have to:

- Create code for an Arduino I2C slave that will be the ‘Watch device’. You might reuse parts of the code.

- Create code for an Arduino I2C master. For this you can of course also reuse part of the already available code.

## Part C (optional): extending the simulator

The watch simulator currently only provides a bare minimum of functionality. It can for instance be extended to support the configuration settings and date functionality.

# Appendix: Watch device registers

## Configuration registers

### 0x20: Configuration register (Read/Write)

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 | Bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - | - | - | - | PS | TR[1] | TR[0] | TF | Function |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | Default |

TF

0: Time format hh:mm

1: Time format hh:mm:ss

TR[1:0]

00: Time refresh disabled

01: Time refresh every 1 second

10: Time refresh every 30 seconds

11: Time refresh every minute

PS[1:0]

0: Time is updated

1: Time updated is paused

## Time registers

The time is stored in registers TIMEH and TIMEL:

### 0x21: Time value register MSB (Read/Write)

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 | Bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HR[3] | HR[2] | HR[1] | HR[0] | MIN[5] | MIN[4] | MIN[3] | MIN[2] | Function |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | Default |

### 0x22: Time value register LSB (Read/Write)

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 | Bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MIN[1] | MIN[0] | SEC[5] | SEC[4] | SEC[3] | SEC[2] | SEC[1] | SEC[0] | Function |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | Default |

The bits in the above registers store the current time:

SEC[5:0]

The number of seconds (0..59)

MIN[5:0]

The number of minutes (0..59)

HR[3:0]

The number of hours (0..11). 
Note: the clock does not support 24-hour time format.

## Date registers

The time is stored in registers DATEH and DATEL:

### 0x23: Date value register MSB (Read/Write)

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 | Bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DM[4] | DM[3] | DM[2] | DM[1] | DM[0] | M[3] | M[2] | M[1] | Function |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | Default |

### 0x24: Date value register LSB (Read/Write)

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 | Bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M[0] | YR [6] | YR [5] | YR [4] | YR [3] | YR [2] | YR [1] | YR[0] | Function |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | Default |

The bits in the above registers store the current date:

YR[6:0]

The number of years since the year 2000 (0..127)

M[3:0]

The month (1..12)

DM[4:0]

The day of month (1..31)