Function watch_registers_toggle_config_is_paused

Introduction:

We consider an 8-bit register named config:

|  | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| config (exemple1) | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| config (exemple2) | 1 | 0 | 1 | 0 | 0 | 0 | 1 | 0 |

We want to toggle b3, i.e. the pause bit. Give the expected content of register config after toggling b3 for the 2 examples given above.

|  | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| config (exemple1) |  |  |  |  |  |  |  |  |
| config (exemple2) |  |  |  |  |  |  |  |  |

Process:

Let’s now review the process to toggle bit b3 of register config.

- Step1: Define an 8-bit mask to manipulate b3.

|  | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| config | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| mask |  |  |  |  |  |  |  |  |

What is the value of mask in hexa-decimal?

- Step2: apply the mask to register config in order to toggle b3. The result is stored in toggled_config.

|  | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| config | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| mask |  |  |  |  |  |  |  |  |
| toggled_config |  |  |  |  |  |  |  |  |

What bit-wise operator will you use to clear b3? Also give the truth table of the corresponding operator.

Function watch_registers_get_config_settings

Introduction:

We consider an 8-bit register named config:

|  | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| config | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |

We want to:
- get/retrieve the ‘time update interval’ placed in bits b2 and b1 of register config
- store the retrieved ‘time update interval’ in a new variable called interval

Give the expected content of variable interval. Tip: place the ‘time update interval’ in the right-most bits of variable interval.

|  | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| interval |  |  |  |  |  |  |  |  |

Process:

Let’s now review the process to get/retrieve the ‘time update interval’ from register config.

- Step1: Define an 8-bit mask to manipulate b2 and b1.

|  | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| config | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| mask |  |  |  |  |  |  |  |  |

- Step2: apply the mask to register config in order to get/retrieve b2 and b1. The result is stored in temp. Tip: you are not expected yet to use a bit-shift operator

|  | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| config | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| mask |  |  |  |  |  |  |  |  |
| temp |  |  |  |  |  |  |  |  |

What bit-wise operator will you use to get/retrieve b2 and b1? Also give the truth table of the corresponding C operator.

- Step3: We now want to place b2 and b1 of temp in the right-most bits of interval. Re-use the binary value of temp found in step2 and give the binary value of interval.

|  | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| temp |  |  |  |  |  |  |  |  |
| interval |  |  |  |  |  |  |  |  |

What bit-wise operator will you use to place b2 and b1 of temp in the right-most bits of interval?