Exercise 4: manipulate a register

Introduction:

We consider an 8-bit register named config:

|  | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| config | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 1 |

We want to set bits b4, b3, b2, b1 of config to 0b1001. So, after bit manipulation, the modified register config will be:

|  | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| config | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 1 |

Process:

Let’s now explore the process to manipulate register config.

- Step1: Define an 8-bit mask to manipulate b4, b3, b2 and b1. Convention: a ‘1’ in the mask indicates that the corresponding bit will be manipulated

|  | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| config | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 1 |
| mask |  |  |  |  |  |  |  |  |

- Step2: Invert the 8-bit mask previously created. The result is stored in inv_mask.

|  | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| config | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 1 |
| inv_mask |  |  |  |  |  |  |  |  |

What bit-wise operator will you use to invert the 8-bit mask? Also give the corresponding C operator.

- Step3: apply the inverted mask to register config in order to clear b4, b3, b2 and b1. The result is stored in cleared_config.

|  | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| config | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 1 |
| inv_mask |  |  |  |  |  |  |  |  |
| cleared_config |  |  |  |  |  |  |  |  |

What bit-wise operator will you use to clear b4, b3, b2 and b1? Also give the corresponding C operator.

- Step4: The 8-bit integer new_value contains 0b1001, the new value to be set in register config. Now, shift new_value to properly match the position of b4, b3, b2 and b1. Result is given in shifted_new_value.

|  | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| new_value | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 |
| shifted_new_value |  |  |  |  |  |  |  |  |

What bit-wise operator will you use to shift b4, b3, b2 and b1? Also give the corresponding C operator.

- Step6: Set the new value of b4, b3, b2 and b1 using cleared_config and shifted_new_value. The result is given in new_config.

|  | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cleared_config |  |  |  |  |  |  |  |  |
| shifted_new_value |  |  |  |  |  |  |  |  |
| new_config |  |  |  |  |  |  |  |  |

What bit-wise operator will you use to set b4, b3, b2 and b1? Also give the corresponding C operator.

Implementation:

Let’s now implement in C a possible solution using the process above and the following prototype. Also, write (at least) one unit test to verify that your algorithm operates as expected. Tip: re-use the starting project from EFIT.

/* * Update the configuration byte by updating the time format. * * @param config: The address of the configuration byte that must be updated. * @param new_val: the new value of bits b4, b3, b2, b1. * * @pre config may not be NULL. */ void watch_registers_set_config_time_format(uint8_t* config, uint8_t new_val);